Unified Drone Control Framework

User Manual

Version 1.0

A reusable, extensible Python framework for unified control
of multiple educational drone platforms

# Table of Contents

1.  Introduction

2.  Architecture Overview

3.  Installation and Setup

4.  Framework Components

5.  Supported Drone Platforms

6.  Quick Start Guide

7.  API Reference

8.  Command Reference

9.  Working with Multiple Drones

10.  Adding a New Drone Platform

11.  Error Handling

12.  Logging and Monitoring

13.  Complete Code Examples

14.  Troubleshooting

# 1. Introduction

The Unified Drone Control Framework is a Python software framework designed to enable unified control of multiple educational drone platforms. It abstracts the differences between heterogeneous drone control libraries, allowing developers and educators to write control logic once and deploy it across different drone hardware without modifying high-level code.

## 1.1 Purpose

Educational drone platforms each provide their own Python libraries with different APIs, naming conventions, and communication protocols. This framework solves the interoperability problem by providing a single, consistent interface that translates generic commands into platform-specific operations.

## 1.2 Key Features

Unified command interface across all supported drone platforms

Adapter pattern isolates platform-specific code from user logic

Open/Closed design: add new drones without modifying existing code

Command pattern with immutable, replayable command objects

Multi-drone orchestration with broadcast and sequencing

Built-in logging at every operation level

Clean exception hierarchy for structured error handling

Zero external dependencies beyond the drone libraries themselves

## 1.3 Supported Platforms

| Platform | Library | Communication |
| --- | --- | --- |
| CoDrone EDU | CoDrone (Robolink) | Bluetooth |
| CodingRider | CodingRider.drone | Bluetooth/Wi-Fi |
| WIZWING | pyserial | Serial (USB) |

## 1.4 Requirements

Python 3.8 or later

Standard library modules: abc, dataclasses, enum, typing, logging, struct

Drone-specific libraries installed only for the platforms you use

# 2. Architecture Overview

The framework is organized into clearly separated layers that ensure loose coupling and high extensibility. Three design patterns form the architectural foundation.

## 2.1 Layer Diagram

User Code / Application

|

v

DroneManager  -->  DroneRegistry  -->  DroneAdapter (ABC)

|

+----------------------------+----------------------------+

|                            |                            |

v                            v                            v

CoDroneEduAdapter            CodingRiderAdapter            WizwingAdapter

(CoDrone library)            (CodingRider.drone)           (pyserial)

## 2.2 Design Patterns

### Adapter Pattern

Each drone library has a dedicated adapter class that translates the unified interface into library-specific API calls. The DroneAdapter abstract base class defines the target interface; concrete adapters (CoDroneEduAdapter, CodingRiderAdapter, WizwingAdapter) implement the translation.

### Command Pattern

Flight operations are represented as immutable DroneCommand data objects that carry an action identifier (FlightAction enum) and a parameter dictionary. This decouples what to do from how to do it and enables command logging, queuing, and replay.

### Factory/Registry Pattern

The DroneRegistry maintains a class-level dictionary mapping string keys to adapter classes. New drones register via the @register_drone decorator. DroneManager uses the registry to instantiate adapters by key, never referencing concrete classes directly. This is how the Open/Closed Principle is enforced.

## 2.3 Package Structure

unified_drone/

__init__.py               # Public API re-exports

core/

__init__.py

enums.py              # FlightAction, Direction, DroneStatus

models.py             # DroneCommand, Position, LEDColor

exceptions.py         # DroneError hierarchy

base_adapter.py       # DroneAdapter abstract base class

registry.py           # DroneRegistry + @register_drone

manager.py            # DroneManager orchestration

adapters/

__init__.py           # Auto-registers all built-in adapters

codrone_edu.py        # CoDrone EDU adapter

coding_rider.py       # CodingRider adapter

wizwing.py            # WIZWING serial adapter

examples/

__init__.py

basic_usage.py        # Unified control demo

add_new_drone.py      # Extensibility demo

## 2.4 Dependency Flow

The core package has zero imports from the adapters package. Adapters import only from core. This one-way dependency ensures that the framework compiles and loads even when no drone libraries are installed.

# 3. Installation and Setup

## 3.1 Framework Installation

Copy the unified_drone/ directory into your project or add its parent directory to your Python path. The framework uses only Python standard library modules and requires no pip installation.

## 3.2 Drone Library Installation

Install only the libraries for the drone platforms you plan to use. Each library is imported lazily (inside the connect() method), so the framework loads successfully even if a library is missing.

### CoDrone EDU

pip install CoDrone

### CodingRider

pip install CodingRider

### WIZWING (pyserial)

pip install pyserial

## 3.3 Verifying the Installation

from unified_drone import DroneManager, DroneRegistry

import unified_drone.adapters

print(DroneRegistry.available())

# Output: ['codrone_edu', 'coding_rider', 'wizwing']

# 4. Framework Components

## 4.1 Enumerations (enums.py)

### FlightAction

Defines all actions supported by the unified interface. Used as the action identifier in DroneCommand objects.

| Value | Description |
| --- | --- |
| CONNECT | Establish connection to the drone |
| DISCONNECT | Close the drone connection |
| TAKEOFF | Command the drone to take off |
| LAND | Command the drone to land |
| EMERGENCY_STOP | Immediately stop all motors |
| MOVE | Move in a specified direction |
| TURN | Rotate by a specified number of degrees |
| HOVER | Hold position for a specified duration |
| SET_LED | Set the drone LED color |
| GET_BATTERY | Query the battery level |
| GET_STATUS | Query the current drone status |

### Direction

Normalized movement directions used by the move() command.

| Value | Description |
| --- | --- |
| FORWARD | Move forward along the drone's heading |
| BACKWARD | Move backward |
| LEFT | Move left (lateral) |
| RIGHT | Move right (lateral) |
| UP | Increase altitude |
| DOWN | Decrease altitude |

### DroneStatus

Lifecycle states tracked by each adapter.

| Value | Description |
| --- | --- |
| DISCONNECTED | No connection to the drone |
| CONNECTED | Connected but not airborne |
| ARMED | Motors armed, ready for takeoff |
| FLYING | Currently airborne |
| LANDING | Landing in progress |
| ERROR | An error has occurred |

## 4.2 Data Models (models.py)

All models are frozen (immutable) dataclasses, making them thread-safe and suitable for logging, queuing, and replay.

### DroneCommand

The central command object. Contains a FlightAction and a dictionary of parameters. Create commands using the convenient factory classmethods.

# Using factory classmethods (recommended)

cmd = DroneCommand.takeoff()

cmd = DroneCommand.move(Direction.FORWARD, distance=2.0, speed=60)

cmd = DroneCommand.turn(degrees=-90)

cmd = DroneCommand.hover(duration=1.5)

cmd = DroneCommand.set_led(LEDColor(red=255, green=0, blue=128))

# Direct construction (advanced)

cmd = DroneCommand(

action=FlightAction.MOVE,

params={"direction": Direction.FORWARD, "distance": 2.0, "speed": 60}

)

### LEDColor

RGB color with brightness for drone LED control.

color = LEDColor(red=255, green=0, blue=0, brightness=100)

# red, green, blue: 0-255

# brightness: 0-100 (default: 100)

### Position

3D position with yaw rotation, for spatial calculations.

pos = Position(x=1.0, y=2.0, z=0.5, yaw=45.0)

## 4.3 Exception Hierarchy (exceptions.py)

| Exception | Raised When |
| --- | --- |
| DroneError | Base exception for all framework errors |
| DroneConnectionError | Connection or disconnection fails |
| DroneCommandError | A command cannot be executed |
| DroneTimeoutError | An operation times out |
| AdapterNotFoundError | No adapter registered for a given key |

from unified_drone import DroneError, DroneConnectionError

try:

manager.send_command("drone_1", DroneCommand.takeoff())

except DroneConnectionError as e:

print(f"Connection issue: {e}")

except DroneError as e:

print(f"Drone error: {e}")

# 5. Supported Drone Platforms

## 5.1 CoDrone EDU

Registry key: "codrone_edu"

Adapter class: CoDroneEduAdapter

Library: CoDrone (Robolink)

The CoDrone EDU adapter wraps the Robolink CoDrone library. It supports Bluetooth pairing with optional port specification, directional movement, turning, hovering, LED control, and battery queries.

### Constructor Parameters

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| name | str | "codrone_edu" | Identifier for the adapter instance |
| port | str ¦ None | None | Serial port for Bluetooth (e.g., COM4) |

### API Translation

| Unified Method | CoDrone EDU API Call |
| --- | --- |
| connect() | CoDrone().pair() or .pair(port) |
| disconnect() | .close() |
| takeoff() | .takeoff() |
| land() | .land() |
| emergency_stop() | .emergency_stop() |
| move(dir, dist, spd) | .go(direction_str, power, duration) |
| turn(degrees) | .turn_right(deg) / .turn_left(abs(deg)) |
| hover(duration) | .hover(seconds) |
| set_led(color) | .set_drone_LED(r, g, b, brightness) |
| get_battery() | .get_battery() |

## 5.2 CodingRider

Registry key: "coding_rider"

Adapter class: CodingRiderAdapter

Library: CodingRider.drone

The CodingRider adapter wraps the CodingRider drone module. Note that CodingRider uses different method names from the unified interface (e.g., landing() instead of land()), which the adapter translates transparently.

### API Translation

| Unified Method | CodingRider API Call |
| --- | --- |
| connect() | drone.connect() |
| disconnect() | drone.disconnect() |
| takeoff() | drone.takeoff() |
| land() | drone.landing() |
| emergency_stop() | drone.landing() (forced) |
| move(dir, dist, spd) | drone.go(dir_code, distance, speed) |
| turn(degrees) | drone.turn(degrees) |
| hover(duration) | time.sleep(duration) (simulated) |
| set_led(color) | Not fully supported (logged) |
| get_battery() | drone.get_battery() if available |

Note: The CodingRider hover() is simulated via time.sleep() because the library does not provide a native hover command. LED control and battery queries may not be available on all CodingRider hardware versions.

## 5.3 WIZWING (pyserial)

Registry key: "wizwing"

Adapter class: WizwingAdapter

Library: pyserial

The WIZWING adapter communicates with the drone via raw serial byte packets. It constructs binary command packets with a defined protocol structure: [HEADER][CMD][LEN][PAYLOAD][CHECKSUM][FOOTER].

### Constructor Parameters

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| name | str | "wizwing" | Identifier for the adapter instance |
| port | str | "COM3" | Serial port (e.g., COM3, /dev/ttyUSB0) |
| baudrate | int | 115200 | Serial communication baud rate |

### Serial Protocol

| Command | CMD Byte | Payload |
| --- | --- | --- |
| TAKEOFF | 0x01 | None |
| LAND | 0x02 | None |
| EMERGENCY | 0x03 | None |
| MOVE | 0x04 | [dir_byte, dist_cm (uint16 BE), speed (uint8)] |
| TURN | 0x05 | [sign_byte, degrees (uint16 BE)] |
| HOVER | 0x06 | [duration_ms (uint16 BE)] |
| LED | 0x07 | [R, G, B, brightness] |
| BATTERY_REQ | 0x08 | None (response contains battery %) |

# 6. Quick Start Guide

## 6.1 Minimal Example

The following example demonstrates the complete workflow: import, register adapters, create a manager, add a drone, and send commands.

from unified_drone import DroneManager, DroneCommand, Direction

import unified_drone.adapters  # triggers adapter registration

# Create the manager

manager = DroneManager()

# Add a drone (choose one of the supported platforms)

manager.add_drone("my_drone", "codrone_edu", port="COM4")

# Connect, fly, and land

manager.send_command("my_drone", DroneCommand.connect())

manager.send_command("my_drone", DroneCommand.takeoff())

manager.send_command("my_drone",

DroneCommand.move(Direction.FORWARD, distance=1.0, speed=50))

manager.send_command("my_drone", DroneCommand.land())

manager.send_command("my_drone", DroneCommand.disconnect())

## 6.2 Switching Drone Platforms

To switch from one drone to another, only the add_drone() call changes. All command logic remains identical.

# Switch from CoDrone EDU to WIZWING - only this line changes:

manager.add_drone("my_drone", "wizwing", port="COM5", baudrate=115200)

# All subsequent commands work exactly the same

manager.send_command("my_drone", DroneCommand.connect())

manager.send_command("my_drone", DroneCommand.takeoff())

# ...

# 7. API Reference

## 7.1 DroneManager

The central orchestration class. Manages multiple drones through the unified interface. Never references concrete adapter classes directly.

| Method | Parameters | Returns | Description |
| --- | --- | --- | --- |
| add_drone() | drone_id, adapter_key, **kwargs | DroneAdapter | Create and register a drone instance |
| remove_drone() | drone_id | None | Remove a drone (disconnects if needed) |
| get_drone() | drone_id | DroneAdapter | Retrieve a managed drone by ID |
| send_command() | drone_id, command | Any | Send a command to one drone |
| broadcast_command() | command | Dict[str, Any] | Send a command to ALL drones |
| send_sequence() | drone_id, commands | List[Any] | Execute a list of commands in order |
| status_report() | (none) | Dict[str, str] | Get status of all drones |
| drone_ids | (property) | List[str] | List all managed drone IDs |

## 7.2 DroneRegistry

Central registry mapping string keys to adapter classes. All methods are classmethods (no instantiation needed).

| Method | Parameters | Returns | Description |
| --- | --- | --- | --- |
| register() | key, adapter_class | None | Register an adapter class under a key |
| get() | key | Type[DroneAdapter] | Look up an adapter class by key |
| create() | key, **kwargs | DroneAdapter | Instantiate an adapter by key (factory) |
| available() | (none) | List[str] | List all registered adapter keys |

## 7.3 DroneAdapter (Abstract Base Class)

The abstract interface all adapters must implement. Contains 10 abstract methods and one concrete dispatch method (execute).

| Method | Parameters | Returns | Description |
| --- | --- | --- | --- |
| connect() | **kwargs | None | Establish drone connection |
| disconnect() | (none) | None | Close drone connection |
| takeoff() | (none) | None | Take off |
| land() | (none) | None | Land |
| emergency_stop() | (none) | None | Stop all motors immediately |
| move() | direction, distance, speed | None | Move in a direction |
| turn() | degrees | None | Rotate (+ = CW, - = CCW) |
| hover() | duration | None | Hold position (seconds) |
| set_led() | color: LEDColor | None | Set LED color |
| get_battery() | (none) | int | Battery % (0-100, or -1) |
| execute() | command: DroneCommand | Any | Dispatch a command (concrete) |

## 7.4 @register_drone Decorator

Class decorator that automatically registers an adapter with the DroneRegistry when its module is imported.

from unified_drone import DroneAdapter, register_drone

@register_drone("my_custom_drone")

class MyCustomAdapter(DroneAdapter):

# implement all abstract methods...

pass

# 8. Command Reference

DroneCommand provides factory classmethods for creating every supported command. All commands are immutable (frozen) dataclasses.

| Factory Method | Parameters | Description |
| --- | --- | --- |
| DroneCommand.connect() | **kwargs (e.g., port) | Connect to drone |
| DroneCommand.disconnect() | (none) | Disconnect from drone |
| DroneCommand.takeoff() | (none) | Take off |
| DroneCommand.land() | (none) | Land |
| DroneCommand.emergency_stop() | (none) | Emergency motor stop |
| DroneCommand.move() | direction, distance=1.0, speed=50.0 | Move in direction |
| DroneCommand.turn() | degrees | Rotate (+CW / -CCW) |
| DroneCommand.hover() | duration=1.0 | Hold position (seconds) |
| DroneCommand.set_led() | color: LEDColor | Set LED color |
| DroneCommand.get_battery() | (none) | Query battery level |
| DroneCommand.get_status() | (none) | Query drone status |

## 8.1 move() Parameters

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| direction | Direction | (required) | FORWARD, BACKWARD, LEFT, RIGHT, UP, DOWN |
| distance | float | 1.0 | Distance in meters |
| speed | float | 50.0 | Speed (0-100 scale) |

## 8.2 turn() Parameters

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| degrees | float | (required) | Positive = clockwise, negative = counter-clockwise |

# 9. Working with Multiple Drones

## 9.1 Adding Multiple Drones

DroneManager supports any number of drones, even of different types. Each drone is identified by a unique string ID.

manager = DroneManager()

# Mix different drone platforms in the same manager

manager.add_drone("edu_1", "codrone_edu", port="COM4")

manager.add_drone("rider_1", "coding_rider")

manager.add_drone("wiz_1", "wizwing", port="COM5")

print(manager.drone_ids)

# ['edu_1', 'rider_1', 'wiz_1']

## 9.2 Broadcasting Commands

broadcast_command() sends the same command to every managed drone. It returns a dictionary mapping each drone ID to the result or exception.

# Connect all drones at once

manager.broadcast_command(DroneCommand.connect())

# Takeoff all drones simultaneously

manager.broadcast_command(DroneCommand.takeoff())

# Land all drones

manager.broadcast_command(DroneCommand.land())

# Check results

results = manager.broadcast_command(DroneCommand.get_battery())

for drone_id, level in results.items():

print(f"{drone_id}: {level}%")

## 9.3 Command Sequences

send_sequence() executes a list of commands on a single drone in order.

flight_plan = [

DroneCommand.move(Direction.FORWARD, distance=2.0, speed=50),

DroneCommand.turn(degrees=-45),

DroneCommand.hover(duration=1.0),

DroneCommand.move(Direction.LEFT, distance=1.0, speed=40),

]

manager.send_sequence("edu_1", flight_plan)

## 9.4 Status Monitoring

# Get status of all drones

print(manager.status_report())

# {'edu_1': 'FLYING', 'rider_1': 'CONNECTED', 'wiz_1': 'FLYING'}

# 10. Adding a New Drone Platform

The framework is designed so that adding a new drone requires exactly one new file and zero modifications to existing code. This section walks through the process using a hypothetical DJI Tello as an example.

## Step 1: Create a New Adapter File

Create a new Python file in the adapters/ directory (or anywhere in your project). Subclass DroneAdapter and implement all 10 abstract methods.

# File: unified_drone/adapters/tello.py

from unified_drone import DroneAdapter, register_drone

from unified_drone.core.enums import Direction, DroneStatus

from unified_drone.core.models import LEDColor

from unified_drone.core.exceptions import DroneConnectionError

@register_drone("tello")

class TelloAdapter(DroneAdapter):

def __init__(self, name: str = "tello") -> None:

super().__init__(name=name)

self._tello = None

def connect(self, **kwargs) -> None:

from djitellopy import Tello

self._tello = Tello()

self._tello.connect()

self._status = DroneStatus.CONNECTED

def disconnect(self) -> None:

if self._tello:

self._tello.end()

self._status = DroneStatus.DISCONNECTED

def takeoff(self) -> None:

self._tello.takeoff()

self._status = DroneStatus.FLYING

def land(self) -> None:

self._tello.land()

self._status = DroneStatus.CONNECTED

def emergency_stop(self) -> None:

self._tello.emergency()

self._status = DroneStatus.CONNECTED

def move(self, direction, distance=1.0, speed=50.0):

dist_cm = int(distance * 100)

mapping = {

Direction.FORWARD:  self._tello.move_forward,

Direction.BACKWARD: self._tello.move_back,

Direction.LEFT:     self._tello.move_left,

Direction.RIGHT:    self._tello.move_right,

Direction.UP:       self._tello.move_up,

Direction.DOWN:     self._tello.move_down,

}

fn = mapping.get(direction)

if fn:

fn(dist_cm)

def turn(self, degrees):

if degrees >= 0:

self._tello.rotate_clockwise(int(degrees))

else:

self._tello.rotate_counter_clockwise(int(abs(degrees)))

def hover(self, duration=1.0):

import time

time.sleep(duration)

def set_led(self, color):

pass  # Tello LED not supported via djitellopy

def get_battery(self):

return self._tello.get_battery()

## Step 2: Import the Module

Import the adapter module so the @register_drone decorator runs. You can either add it to adapters/__init__.py for automatic registration, or import it directly in your script.

# Option A: Add to adapters/__init__.py for auto-registration

from . import tello  # add this line

# Option B: Import directly in your script

import unified_drone.adapters.tello  # triggers registration

## Step 3: Use It

# The "tello" key is now available

print(DroneRegistry.available())

# ['codrone_edu', 'coding_rider', 'wizwing', 'tello']

manager = DroneManager()

manager.add_drone("my_tello", "tello")

manager.send_command("my_tello", DroneCommand.connect())

manager.send_command("my_tello", DroneCommand.takeoff())

# ... all unified commands work transparently

Note: Zero existing files were modified. The new adapter registered itself through the decorator, and the manager discovered it through the registry. This is the Open/Closed Principle in practice.

# 11. Error Handling

The framework uses a structured exception hierarchy rooted at DroneError. All drone-related exceptions can be caught with a single except DroneError clause, or handled specifically by type.

## 11.1 Exception Hierarchy

DroneError (base)

DroneConnectionError    # connection/disconnection failures

DroneCommandError       # command execution failures

DroneTimeoutError       # timeout during operations

AdapterNotFoundError    # unregistered adapter key

## 11.2 Handling Errors

from unified_drone import (

DroneError, DroneConnectionError,

DroneCommandError, AdapterNotFoundError

)

try:

manager.add_drone("drone_1", "unknown_platform")

except AdapterNotFoundError as e:

print(f"Platform not found: {e}")

try:

manager.send_command("drone_1", DroneCommand.takeoff())

except DroneConnectionError:

print("Drone is not connected")

except DroneCommandError as e:

print(f"Command failed: {e}")

except DroneError as e:

print(f"General drone error: {e}")

## 11.3 Broadcast Error Handling

broadcast_command() does not raise exceptions. Instead, it catches errors per-drone and includes them in the result dictionary.

results = manager.broadcast_command(DroneCommand.takeoff())

for drone_id, result in results.items():

if isinstance(result, DroneError):

print(f"{drone_id} failed: {result}")

else:

print(f"{drone_id} succeeded")

# 12. Logging and Monitoring

Every component in the framework uses Python's built-in logging module. Adapters log all operations at appropriate levels.

## 12.1 Enabling Logging

import logging

# Basic setup - see all framework messages

logging.basicConfig(

level=logging.INFO,

format="%(name)s | %(levelname)s | %(message)s"

)

# Or configure specific loggers

logging.getLogger("unified_drone.adapters.codrone_edu").setLevel(logging.DEBUG)

## 12.2 Log Levels Used

| Level | Usage |
| --- | --- |
| DEBUG | Raw serial data (WIZWING TX bytes), internal dispatch |
| INFO | Successful operations (connect, takeoff, move, land) |
| WARNING | Emergency stops, unsupported features, adapter overwrites |
| ERROR | Failed operations during broadcasts, disconnection errors |

# 13. Complete Code Examples

## 13.1 Multi-Platform Flight Demo

This example manages three different drones simultaneously, demonstrating broadcasting, individual commands, and sequences.

import logging

from unified_drone import (

DroneManager, DroneCommand, Direction,

DroneRegistry, LEDColor

)

import unified_drone.adapters

logging.basicConfig(level=logging.INFO, format="%(name)s | %(levelname)s | %(message)s")

# Show registered adapters

print(DroneRegistry.available())

# Create manager and add drones

manager = DroneManager()

manager.add_drone("edu_1", "codrone_edu", port="COM4")

manager.add_drone("rider_1", "coding_rider")

manager.add_drone("wiz_1", "wizwing", port="COM5")

# Connect and takeoff all

manager.broadcast_command(DroneCommand.connect())

manager.broadcast_command(DroneCommand.takeoff())

# Individual commands

manager.send_command("edu_1",

DroneCommand.move(Direction.FORWARD, distance=1.5, speed=60))

manager.send_command("rider_1", DroneCommand.turn(degrees=90))

manager.send_command("wiz_1",

DroneCommand.set_led(LEDColor(red=255, green=0, blue=0)))

# Battery check

batteries = manager.broadcast_command(DroneCommand.get_battery())

for drone_id, level in batteries.items():

print(f"  {drone_id}: battery = {level}%")

# Command sequence on one drone

flight_plan = [

DroneCommand.move(Direction.FORWARD, distance=2.0, speed=50),

DroneCommand.turn(degrees=-45),

DroneCommand.hover(duration=1.0),

DroneCommand.move(Direction.LEFT, distance=1.0, speed=40),

]

manager.send_sequence("edu_1", flight_plan)

# Land and disconnect all

manager.broadcast_command(DroneCommand.land())

manager.broadcast_command(DroneCommand.disconnect())

print(manager.status_report())

## 13.2 Reusable Flight Logic

Because the interface is unified, you can write functions that accept a drone ID and a manager, then work with any platform.

def square_flight(manager, drone_id, side_length=1.0, speed=50):

"""Fly a square pattern. Works with any drone platform."""

for _ in range(4):

manager.send_command(drone_id,

DroneCommand.move(Direction.FORWARD,

distance=side_length, speed=speed))

manager.send_command(drone_id,

DroneCommand.turn(degrees=90))

# Works identically on any drone:

square_flight(manager, "edu_1", side_length=2.0)

square_flight(manager, "wiz_1", side_length=1.5)

# 14. Troubleshooting

| Issue | Cause | Solution |
| --- | --- | --- |
| AdapterNotFoundError | Adapter module not imported | Add 'import unified_drone.adapters' before using the manager |
| DroneConnectionError on connect | Wrong port or drone not powered on | Verify the serial port and ensure the drone is turned on |
| DroneConnectionError on command | connect() was not called first | Always call DroneCommand.connect() before other commands |
| get_battery() returns -1 | Platform does not support battery query | Check adapter documentation for platform limitations |
| LED commands have no effect | Platform does not support LED control | CodingRider LED support is limited; check hardware version |
| ImportError for drone library | Library not installed | Install the specific library: pip install CoDrone / pyserial / etc. |
| Framework loads but no adapters | adapters/__init__.py not imported | Ensure 'import unified_drone.adapters' is in your code |

End of Manual